| FPVS Analysis LetsWave6 Download & SetUp Letswave is a free software toolbox based on MATLAB to analyze electroencephalography (EEG), working on Windows, Mac OSX, and Linux or Unix. Unzip the file you download into the folder of your choice. Add the folder into the path of Matlab. Input “letswave6” in the command window of Matlab to check whether the install is successful or not. For software and hardware requirements: https://letswave.cn/gs_general_info.html Analysis Import Raw Data Raw EEG data has 3 types of data: .eeg, .vhdr, and .vmrk. To import .eeg file: File -> Import Signals -> Import Neuroscan EEG During the importing, LetsWave will display “processing”. Once the importing is finished, the corresponding dataset will turn bold or red and display “done”. After each step you have done, you can see ***Finished Process in the command window of letswave itself. Brain Signals Brainwave Frequencies: EEG measures the electrical activity of the brain, which consists of different types of brain waves associated with different mental states and activities. The frequency range of interest typically includes different brainwave bands: Delta (0.5 - 4 Hz) associated with deep sleep and unconsciousness. Theta (4 - 8 Hz) associated with drowsiness, relaxation, and light sleep. Alpha (8 - 13 Hz) associated with relaxed wakefulness and closing of the eyes. Beta (13 - 30 Hz) associated with active thinking, concentration, and alertness. Gamma (30 - 100 Hz) associated with higher cognitive processes and sensory integration. The range up to 100 Hz covers most of the relevant brainwave frequencies and certain event-related potentials, (and in our case, SSVEPs) researchers may also focus on narrower frequency bands or investigate higher-frequency components in specific contexts or applications. Cheatsheet - Preprocessing |  |  |  |
| --- | --- | --- | --- |
| Order | Step | Prefix | Remark |
| 1 | Rename channels. | chanlabels | Edit > Electrodes > Edit electrode labels EXT1 – I1 EXT2 – I2 EXT3 – PO9 EXT4 – PO10 /!\ Don’t forget to replace dead electrodes /!\ |
| 2 | Electrodes location. | chanlocs | Edit > Electrodes > Edit electrode coordinates biosemi_locations_64_fixP9P10_add4.xyz. |
| 3 | Remove unused channels. | chan-select | Preprocess > Arrange signals > Rearrange / Delete channels Status and unused externals (5-6). |
| Optionnal | Check triggers. | NA | Events > Browse and edit events Remove repeated events. If missing triggers, check if new unusual triggers and verify with the same method as for duplicated triggers. Then, either change the name directly (if all are missing) or add a new one in the correct triggers and copy-paste its delay. For instance, in my analysis, I was often missing a fade-in trigger for one condition. To find it back, I went to the fade-out events of the same condition and calculated the time of the fade-in (subtract 68sec) and go to other triggers and find the one that appears at the same time as the one I’m looking for. Update event list and save after each modification. |
| 4 | Butterworth filter. | but | Preprocess > Frequency filters > Butterworth filter 0.0.05Hz – 100Hz. 4 |
| 5 | Notch filter. | notchfilt | Preprocess > Frequency filters > FFT multi-notch filter The notch filter in the preprocessing step using the Butterworth filter is commonly used to eliminate unwanted interference or noise that is present at a particular frequency. The frequency of 50 or 60 Hz is typically associated with the AC (alternating current) power mains frequency used in electrical power distribution systems in different regions of the world. Notch filter frequency 50Hz-100Hz, width 0.05Hz and slope 2Hz. Unclick “inverse filter”. |
| 6 | Downsampling | ds | Preprocess > Resample signal > Downsample signal (integrer ratio) Downsampling is commonly used to reduce data size, computational requirements, and to focus on specific frequency bands while preserving essential information in the signal. It reduces the sampling rate of the signal. In this process, some of the original samples are discarded, resulting in a lower sampling rate. The Nyquist-Shannon Sampling Theorem states that the sample rate must be at least twice the bandwidth of the signal to avoid aliasing distortion. In practice, because of the finite time available, a sample rate somewhat higher than this is necessary. Remember that we are interested in frequencies that are in a range up to 100 Hz. Therefore, if we downsample it by 8, we will see a signal until 125 Hz, which covers up our range of interest. Here we record at 512. 256Hz is plenty sufficient so we divide by 2. Know that you can only look at frequency twice as small as the sampling frequency maximum. Here with 256, that would be 128Hz. |
| 7 | Segmentation of events. | ep | Preprocess > Segmentation > Segmentation relative to event 2 sec before start + fade in (2 sec) + block duration (64 sec) + fade out (2) = 70 sec Start: -2 Duration : 70 Choose triggers corresponding to beginning of events. |
| Optionnal | Compute ICA. | ica | Plugins > Blink detection and count Note that ICA can create distortions in the data, should always compare to data without ICA applied. Select the electrodes that show the blink well. Either the external ones on the eye but Fp2 also works really well for instance A document will be created in the folder. To read it better you can copy paste it in an excel file. It gives you the number of blinks per epoch and the number of blinks per second. It is considered that you need blink correction if the number of blinks exceeds 0.2 per second. This is not an absolute rule, you might also need to do it if it less but on each epoch for instance. To do eye correction Preprocess > Spatial filters > Compute ICA matrix (don’t change parameters RUNICA and SQUARE) It decomposes the signal into matrices. Preprocess > Spatial filters > ICA apply spatial filter Matrices are on top, and below is your signal. You need to choose the matrix that has the blinks. After this step, you can either delete your external channels or keep them but remember not to take them in account |
| Optional | Interpolation | Chan-interp | Edit > Electrodes > Interpolate with neighbouring channels To check if a channel needs interpolating: look at the signal by taking 4/5 neighbouring electrodes. If one is weird on more than 20% of the signal then interpolate No more than 5% of channels /!\ Do not interpolate a channel with a channel that you will also interpolate or already did. To select more, write 4 or 5 channels and untick the ones you don’t want to arrive at 3. |
| 8 | Rereference | reref | Preprocess > Rereference Select all the channels that were used (except for the eyes and problematic channels that were not interpolated) on both sides. |
| - Postprocessing |  |  |  |
| 9 | Segmentation with exact number of bins. One file per condition /!\ | epbin | Preprocess > Segmentation > Segmentation relative to event (one file per event) To know the number of bins, use the function integecycle in matlab. integercycle(StimFreq,SamplingRate,SegmentLength) Here the stimFreq is thr smallest one, so the oddball one. We stim at 6hz and every 5 image is a face, so 1.2Hz Sampling rate is 256 Segmentlength is 64 16213bins (for 0.6 and 1.2hz / 16384 for 0.75 hz), start at 2 This is now the time to merge epochs of different datasets if you need to. |
| 10 | Fourier transformation. | fft | Preprocess > Frequency Transform > FFT Amplitude Normalize (divide by n) After this; don’t hesitate to look at the data. There should be dips at 50Hz and 100Hz. (know that the lower frequencies are always very noisy) |
| 11 | Average trials within the same condition | avg | Postprocess > Average > Average epochs Can be done on all files simultaneously. Letswave checks epochs within the same file. |
| 12 | Chunks | chunk | Preprocess > Segmentation > Segmentation of epochs in successive chunks This step is to be done before any baseline correction. Make chunks around the frequencies of interest (oddball + harmonics) From Talia’s doc Onset: 1.0037 Duration: 0.394671868 Interval: 1.201 If you gather all the electrodes and put the cursor right at the middle of the X axis (often 0.18948), you should see a peak in the fifth epochs and following harmonics. |
| 13 | Select chunks harmonics | ep-select | Preprocess > Arrange signal > Rearrange/delete epochs You have chunks for every harmonic available in your data. Now you want to select about 12 harmonics at the oddball frequency, excluding the base frequencies and it’s harmonics. Ep1= 1.2, Ep2 = 2.4, Ep3 = 3.6, Ep4 = 4.8, Ep5 = 6, Ep6 = 7.2, Ep7 = 8.4, Ep8 = 9.6, Ep9 = 10.8, Ep10 = 12, Ep11 = 13.2, Ep12 = 14.4, Ep13 = 15.6, Ep14 = 16.8, Ep15 = 18… Save it with either oddball or baseline in the name to find it later |
| 14 | Sum harmonics | avg | Postprocess > Average > Average epochs Select “sum” in the menu You are now ready to do the baseline correction |
| Optional | SNR and bsl corrections | snr or bsl | Preprocess > Baseline operations > SNR Preprocess > Baseline operations > Baseline correction snr = choose SNR. bsl = choose Substract. Range 2-12, 1 extreme bin to remove. |
| Optional | Compute sum of harmonics | sumH#-# | Extract y-data at fixed x-intervals. Compute frequency of interests + harmonics multiplier (e.g., 1 2 3 4 = harmonics from 1 to 4). |
| Optional | Build topographies | NA | 1 map = sum of harmonics, # maps = each harmonic individually. X start/end = frequency of interest or first/last harmonic. Color scale = 0 and empty. Put electrodes on. /!\ Put conditions (in terms of stimuli) together, otherwise scale will be biased for conditions with smaller range of data points. |

Steps in red indicate transformation of the data